BRIGHTLINGSEA HARBOUR COMMISSIONERS

INVITATION TO TENDER

5 METRE ALUMINIUM CHAIN / WORK BARGE

1. INTRODUCTION

Brightlingsea Harbour Commissioners are seeking quotations from suitably experienced boatbuilders for the design and construction of a new 5 metre aluminium chain / work barge.

The vessel will be used by Brightlingsea Harbour for general harbour maintenance and operational duties, including:

- Mooring and chain work

- Buoy and Aids to Navigation maintenance

- Pontoon and marina maintenance

- Handling and movement of equipment

- Lifting operations

- General harbour maintenance

- Transporting personnel and equipment around the harbour

The vessel is intended to be a robust, practical working platform capable of operating regularly in a harbour environment.

2. BASIC REQUIREMENTS

| Requirement | Specification |
| --- | --- |
| Overall length | Approximately 5 metres |
| Beam | Approximately 2 metres |
| Hull construction | Aluminium |
| Hull form | Monohull or catamaran (catamaran to include removable bow sheet) |
| Propulsion | Yamaha F25 Outboard |
| Intended use | Harbour maintenance / chain and mooring work |
| Working load | Approximately 1 tonne |
| Winch | 12 V electric winch – mounting provision only |
| Lifting frame | Removable centre lifting frame |
| Bow roller | Heavy-duty |
| Wheelhouse | Tender option required with and without |
| Guarding | Perimeter railings |
| Deck wash | Skin fitting / connection point for deck wash pump |
| Electrical system | 12 V – mounting provision only |
| Target budget | £15,000 |
| Required delivery | April / May 2027 |

The final dimensions may be adjusted by the builder where necessary to achieve a safe, practical and economical design, subject to approval by Brightlingsea Harbour Commissioners.

3. HULL CONSTRUCTION

The barge is to be constructed from marine-grade aluminium suitable for the intended operating environment.

The builder is to specify:

- Aluminium grade and thicknesses

- Hull construction method

- Frame spacing and construction

- Deck construction

- Transom construction

- Internal strengthening

- Welding specification

- Watertight compartments / buoyancy arrangements

- Estimated lightship weight

The hull should be designed as a robust working vessel rather than a recreational craft.

The builder should allow for regular loading and unloading of equipment and the handling of mooring chain and associated equipment.

4. DECK

The working deck should provide as much clear and usable working space as reasonably possible.

The deck should be suitable for handling:

- Mooring chain

- Shackles

- Buoys

- Ropes

- Harbour equipment

- Maintenance equipment

The deck should have a suitable non-slip finish.

Particular attention should be given to preventing equipment, chain and mooring lines from becoming trapped or snagged around the bow.

5. WORKING LOAD

The vessel is required to have a working load capability of approximately:

1,000 kg / 1 tonne

The builder is to advise the maximum safe working load of the vessel and should clearly state:

- Maximum deck loading

- Maximum lifting capacity, if applicable

- Maximum safe operating conditions

- Any limitations on loading, lifting, carrying equipment or load position

The vessel must remain stable and safe when carrying its intended working loads.

6. OUTBOARD ENGINE

The vessel is to be designed for a Yamaha F25 outboard engine.

(mounting provision only)

The tender should include:

- Suitable reinforced outboard mounting arrangement and transom construction

- Engine controls

- Fuel system / fuel storage requirements

- Battery installation

- Steering system

7. 12 VOLT WINCH

A 12 V electric winch is required for future chain and mooring operations.

The tender is to include winch mounting provision only: a suitably engineered mounting plate / mounting structure for the winch.

The winch itself does not need to be supplied unless separately priced as an option.

The mounting arrangement must:

- Be securely attached to the vessel's structure

- Be suitable for the anticipated winch loads

- Allow straightforward installation of the winch

- Provide suitable access for maintenance

- Avoid compromising the working deck

The builder should identify the proposed mounting position and provide details of the design load used.

8. BOW ROLLER

A heavy-duty bow roller is required.

The roller will be used for handling:

- Mooring chain

- Rope

- Buoy equipment

- Other harbour equipment

The roller should be robust and suitable for regular working use.

The builder should specify the proposed roller dimensions, construction and safe working load.

9. REMOVABLE BOW SHEET

Where the vessel is constructed as a catamaran, a removable bow sheet / cover is required.

The purpose of the sheet is to prevent mooring chains and other equipment from becoming trapped underneath or between the forward sections of the vessel when recovering chain onto the deck.

The sheet should:

- Be robust

- Be safely secured during use

- Not interfere with the bow roller

- Allow straightforward access for maintenance

10. CENTRE LIFTING FRAME

A removable centre lifting frame is required.

The frame is intended for lifting and handling equipment from the water and onto the working deck.

Approximate required height:

7 ft / 2.1 metres

The final dimensions should be agreed with Brightlingsea Harbour Commissioners.

The frame should:

- Be securely located when installed

- Be suitably reinforced

- Allow safe lifting of the intended loads

- Not unnecessarily restrict deck space

- Be designed with suitable lifting points where required

The builder must state the Safe Working Load (SWL) of the lifting frame.

11. RAILINGS / GUARDING

Continuous protective railings are required around the working areas of the vessel.

The railings should provide protection for personnel while allowing practical access to the working deck.

The design should consider:

- Access points

- Chain handling

- Mooring operations

- Loading and unloading

- Lifting operations

- Personnel movement around the deck

The builder should identify any areas where railings cannot reasonably be fitted because of operational requirements.

12. WHEELHOUSE

Two prices are required.

OPTION 1 – WITH WHEELHOUSE

Provide a compact wheelhouse suitable for operating the vessel while carrying out harbour work.

The wheelhouse should provide:

- Protection from weather

- Suitable visibility

- Helm position

- Steering controls

- Engine controls

- Suitable seating / standing position

- Windscreen

- Wiper where appropriate

- Suitable access

The wheelhouse should be kept as compact as practical to maximise usable working deck space.

OPTION 2 – WITHOUT WHEELHOUSE

Provide the same vessel specification without a wheelhouse.

The helm / controls are to be arranged appropriately for open-deck operation.

The tender should clearly identify the additional cost of the wheelhouse option.

13. DECK WASH

Provision is required for a deck wash system.

The builder is to provide a suitable skin fitting / connection point for a deck wash pump.

The pump itself may be supplied separately by Brightlingsea Harbour Commissioners.

The builder should identify the proposed location and ensure the installation does not compromise hull integrity or deck strength.

14. ELECTRICAL SYSTEM

A suitable 12 V electrical system is required.

(mounting provision only)

The system should provide for:

- Battery installation

- Battery isolation

- Navigation lights

- Working lights

- Winch connection

- Engine starting

- Deck wash pump connection

- Spare capacity for future equipment

The builder should provide a suitable main battery isolation arrangement.

15. SAFETY

Safety is a primary consideration in the design and construction of the vessel.

The builder should consider:

- Stability

- Freeboard

- Guarding

- Safe access

- Working around the bow

- Chain handling

- Lifting operations

- Manual handling

- Outboard installation

- Emergency access

- Crew safety

The builder must identify any operational limitations or safety considerations associated with the proposed design.

16. PAINTING / FINISH

The aluminium hull and structure should be suitably prepared and finished for the intended marine environment.

The builder should specify:

- Surface preparation

- Coating system

- Anti-slip deck finish

- Any required antifouling system

- Expected coating life

- Maintenance requirements

Any areas where aluminium is in contact with other metals should be suitably protected against corrosion.

17. TRAILER / TRANSPORT

Please provide an optional price for a suitable trailer or transport solution if available.

This should be priced separately from the main vessel in the tender price breakdown (Section 23).

Trailer required: Yes / No / Optional

18. DOCUMENTATION

On completion, the builder is to provide relevant documentation including, where applicable:

- Construction drawings

- General arrangement drawing

- Electrical diagram

- Engine information

- Equipment manuals

- Maintenance information

- Lifting frame information

- Winch mounting information

- Safe Working Loads

- Vessel weight

- Maximum recommended loading

- Warranty information

19. TESTING AND HANDOVER

Before handover, the vessel should be inspected and tested.

The builder should demonstrate:

- Engine operation

- Steering

- Electrical systems provision

- Navigation lights provision

- Working lights provision

- Winch mounting provision

- Lifting frame installation

- Bow roller

- Deck wash connection provision

- General watertight integrity

- General stability and handling

Any defects identified during the handover inspection should be rectified prior to final acceptance.

20. WARRANTY

The tenderer is to state the warranty provided for:

- Hull

- Welding

- Structure

- Equipment supplied

- Paint / coating system

Please provide the warranty period and details of any exclusions.

21. BUILD PROGRAMME

The required delivery period is:

April / May 2027

Tenderers are requested to provide:

Estimated build time: ______________________

Estimated start date: ______________________

Estimated completion date: _________________

Any factors that could affect the delivery programme should be identified in the tender response.

22. SITE / DESIGN MEETING

The successful tenderer may be required to attend a meeting with Brightlingsea Harbour Commissioners to discuss the final design before construction begins.

The final design will be subject to approval before construction.

No significant changes to the agreed specification are to be made without prior agreement.

23. TENDER PRICE

Please provide a clear breakdown of costs.

| Item | Price |
| --- | --- |
| Base 5 m aluminium barge | £ |
| Outboard mounting / controls | £ |
| 12 V winch mounting plate | £ |
| Bow roller | £ |
| Removable lifting frame | £ |
| Perimeter railings | £ |
| Wheelhouse option | £ |
| Deck wash skin fitting | £ |
| Trailer – optional | £ |
| Other | £ |
| TOTAL | £ |

All prices should clearly state whether VAT is included or excluded.

24. ALTERNATIVE PROPOSALS

Builders are encouraged to identify alternative designs or construction methods that may provide:

- Improved durability

- Improved safety

- Reduced maintenance

- Improved working capacity

- Reduced cost

- Improved performance

Any alternative proposal must clearly identify how it differs from this specification.

25. TENDER RESPONSE

Tender submissions should include:

- The completed Tender Application Form at the end of this document, including company details, relevant boatbuilding experience, previous projects and references (where available)

- Proposed vessel design

- General arrangement drawing, where available

- Construction specification

- Full price breakdown, including optional costs (Section 23)

- Proposed engine specification

- Proposed build programme (Section 21)

- Warranty details

- Any deviations from this specification

26. BUDGET

The target budget for the project is:

£15,000

This is a target budget rather than a fixed specification limit.

Tenderers should provide their most competitive price and clearly identify any items that are excluded from the base price.

Where the complete specification cannot be achieved within the target budget, tenderers should provide:

A. The closest specification achievable within £15,000

and

B. The cost of the complete requested specification.

Tender Application Form

5 Metre Aluminium Chain / Work Barge

Below is the tender application form for the design and construction of the 5 metre aluminium chain / work barge described in this specification.

The purpose of this application form is for the Harbour to assess the suitability and eligibility of each tenderer. Please complete all sections and return the form with your tender submission (Section 25), continuing on a separate sheet where needed. Where "Evidence" is shown in the Remark column, please attach supporting documents.

|  | Required Info | Detail | Remark |
| --- | --- | --- | --- |
| Company Information: |  |  |  |
| 1. | Business Name |  |  |
| 2. | Company Registration Number |  |  |
| 3. | Trading Address |  |  |
| 4. | Contact Information | Contact name: Email: Telephone: Address (if different from trading address): |  |
| Financial Standing: |  |  |  |
| 5. | VAT Number |  |  |
| 6. | UTR Number |  |  |
| 7. | Any past or ongoing financial issues |  |  |
| 8. | Please provide details of any current or previous financial, legal or other disputes with Brightlingsea Harbour Commissioners |  |  |
| Insurance Details: |  |  |  |
| 9. | Public and Products Liability |  |  |
| 10. | Employers' Liability |  |  |
| 11. | Professional Indemnity |  |  |
| 12. | Any Other Relevant Insurance Information | e.g., contract works / builder's risk cover for the vessel during construction, launch and delivery. |  |
| Accreditations and Memberships: Relevant industry certifications (e.g., ISO 9001 for quality management, ISO 14001 for environmental management, CHAS for health and safety) and memberships of relevant trade associations (e.g., British Marine, Workboat Association). |  |  |  |
| 13. | Name | Accreditations and Memberships |  |
| 14. | Name | Accreditations and Memberships |  |
| 15. | Name | Accreditations and Memberships |  |
| 16. | Name | Accreditations and Memberships |  |
| Experience and Past Performance: |  |  |  |
| 17. | Project Experience | Detailed case studies of comparable vessels built (e.g., aluminium workboats or work barges), including the scope of work, value and client. Summarise them in Annex Table 1. |  |
| 18. | Relevant Experience | Experience with similar construction methods (marine aluminium fabrication and welding), equipment (e.g., lifting frames, bow rollers, winch mountings, wheelhouses) and harbour / estuary operating conditions. |  |
| 19. | Personnel Expertise | Information on key personnel involved in the project, including their qualifications, experience, and roles. A separate list is required with names, roles, qualifications (including welding qualifications) and dates (from and to). See Annex Table 2. |  |
| Health and Safety: |  |  |  |
| 20. | Health and Safety Policies | Detailed policies and procedures related to health and safety, including compliance with relevant legislation (e.g., Health and Safety at Work etc. Act 1974). Attach list of policies | Evidence |
| 21. | Accident / Incident Records | Details of any accidents, incidents, or near-miss reports in the last 12 months, as well as safety-related awards or recognition. | Evidence |
| 22. | Risk Assessments | Information on how the tenderer identifies and manages risks associated with vessel construction, launching, sea trials and delivery. Attach examples of RAMS | Evidence |
| Technical Capability and Resources: |  |  |  |
| 23. | Facilities and Equipment | Details of the workshop, fabrication, welding, lifting and launching facilities owned by or available to the tenderer, including any work to be subcontracted. | Evidence |
| 24. | Technical Staff and Design | Information on the number and qualifications of technical personnel available for the project, and who will carry out the vessel design, structural calculations and stability assessment (in-house or external naval architect). | Evidence |
| 25. | Quality Management System | Information on the tenderer's quality control procedures and processes (e.g., material certification and traceability, weld inspection, dimensional and watertightness checks). | Evidence |
| Sustainability and Environmental Policies: |  |  |  |
| 26. | Environmental Management System | Details of the tenderer's environmental management system and any relevant certifications (e.g., ISO 14001). | Evidence |
| 27. | Waste Management Procedures | Information on how the tenderer manages fabrication waste (e.g., aluminium offcuts and swarf, paints and solvents) and other waste materials in an environmentally responsible manner. |  |
| 28. | Environmental Impact Mitigation | Details of the measures taken to minimise environmental impact during construction, painting, launching and delivery of the vessel. |  |
| Other Considerations: |  |  |  |
| 29. | References (where available) | Contact details of previous clients who can provide references for the tenderer. Names and, if available, written statements. |  |
| 30. | Miscellaneous | Anything believed to be of further relevance. |  |
| Declaration: |  |  |  |
| We confirm that we have reviewed the above specification and understand the requirements of the proposed vessel, and that the information given in this application is accurate and complete. |  |  |  |
| Tender Price – Option 1 (with wheelhouse) |  | £ |  |
| Tender Price – Option 2 (without wheelhouse) |  | £ |  |
| Alternative Price within £15,000 (Section 26 A, if applicable) |  | £ |  |
| VAT (included / excluded) |  |  |  |
| Proposed Delivery Date |  |  |  |
| Tender Valid Until |  |  |  |
| Name |  |  |  |
| Position |  |  |  |
| Signature |  |  |  |
| Date |  |  |  |

Annex Table 1 – Comparable Vessels Built

Summarise comparable vessels built (Question 17). Add rows as required.

| Ser | Vessel Name / Type | Length × Beam | Hull Material | Client | Year Delivered | Code / Certifying Authority | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- |
| xx | e.g., 6 m aluminium workboat | 6.0 m × 2.4 m | Marine-grade aluminium | Harbour authority | 2025 | Workboat Code Edition 3 / YDSA | Reference available |
| 01 |  |  |  |  |  |  |  |
| 02 |  |  |  |  |  |  |  |
| 03 |  |  |  |  |  |  |  |
| 04 |  |  |  |  |  |  |  |
| 05 |  |  |  |  |  |  |  |
| 06 |  |  |  |  |  |  |  |
| 07 |  |  |  |  |  |  |  |
| 08 |  |  |  |  |  |  |  |

Annex Table 2 – Personnel Qualifications

List key personnel and their qualifications (Question 19). Add rows as required.

| Ser | Name | Role | Qualification | From | Expires | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| xx | Person 1 | Lead welder | Aluminium welding (BS EN ISO 9606-2) | Jan 2025 | Jan 2027 | Certificate provided |
| 01 |  |  |  |  |  |  |
| 02 |  |  |  |  |  |  |
| 03 |  |  |  |  |  |  |
| 04 |  |  |  |  |  |  |
| 05 |  |  |  |  |  |  |
| 06 |  |  |  |  |  |  |
| 07 |  |  |  |  |  |  |
| 08 |  |  |  |  |  |  |
| 09 |  |  |  |  |  |  |